Texas Instruments

TIDA-050056 Test Procedure

Rev. E1

Dec, 9, 2021

# GENERAL

## PURPOSE

The purpose of this document is to provide detailed instructions for testing the

TIDA-050056 module

## SCOPE

This document covers complete instructions for testing TIDA-050056. Upon completion of this procedure, all units will be categorized as either conforming or non-conforming material and dispositioned accordingly.

## REFERENCE DOCUMENTATION

- Schematic TIDA-050056

- TIDA-050056 Assembly Drawings

## MATERIALS

### Test Log: A test log is not required for this procedure.

## DEFINITIONS

N/A

# SAFETY

## EYE PROTECTION

Safety Glasses are to be worn while performing all testing on the TIDA-050056.

## GENERAL RISKS

This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

## ELECTROSTATIC DISCHARGE

ESD precautions must be followed while handling electronic assemblies.

## THERMAL/SHOCK HAZARDS

Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

# QUALITY

## DATA PRESERVATION

Recorded test data is not required for this procedure.

# APPAREL

## ELECTROSTATIC SMOCK

## ELECTROSTATIC GLOVES OR FINGER COTS

## SAFETY GLASSES

## GROUNDED ESD WRIST STRAP

# EQUIPMENT

## POWER SUPPLIES

A power supply capable of supplying 12 volts @ 1 amps is required to supply Vin.

## LOAD

One electronic load capable of sinking 1 Amp @ 3.3 V. This electronic load must have a resistive load setting.

## METERS

Two DC voltmeters are required.

# EQUIPMENT SETUP

## INPUT SUPPLY

Set the input voltage power supply to 12 V and the current limit to 1 A.  Disable the power supply. Connect the input power supply to J1 between VIN (+) and GND (-)

of TIDA-050056.

## LOAD

The load should be connected to TIDA-050056 on J2 between VOUT (+) and GND (-). If using an electronic load, make sure it is set to resistive or constant resistance mode.

## METERS

For the input voltage, connect the positive lead of the first voltmeter to the Vin(+) connection on the J1 connector and the negative terminal of the voltmeter to the GND(-) connection on the J1 connector.

For the output voltage, connect the positive terminal of the second voltmeter to the Vout(+) connection on the J2 connector and the negative terminal of the voltmeter to the GND(-) connection on the J2 connector.

# PROCEDURE

## STEP ONE:  APPLY POWER AND MEASURE VOUT AT NO LOAD

Ensure that the input power supply current limit is set at 1 Amps.  Ensure that the load current is set to 0A.  Using the input power supply, apply the 12 V input voltage (VIN) at J1.  Verify the input voltage at J1 is within 11.5 V to 12.5 V.  Verify the output voltage at J2  is within 3.24V to 3.42V.

## STEP TWO: SET LOAD CURRENT AND MEASURE VOUT

Adjust the load current to 1A

Verify the input voltage measured at J1 is within 11.5 V to 12.5 V. Verify the output voltage at J2 is within 3.24V to 3.42V

## STEP THREE: POWER OFF

Once these measurements are complete, power off the input supply and disconnect the unit under test.

## STEP FOUR: MATERIAL DISPOSITION

All units (passed and failed) shall be dispositioned according to section 9.  Continue testing by going back to step one (section 7.1) until all units have been tested.

# EQUIPMENT SHUTDOWN

N/A

# MATERIAL DISPOSITION & TRANSFER

## CONFORMING MATERIAL

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled on the ESD bag per Table 1, and shipped per the P.O.  The ESD bag shall have both labels.

On the ESD bag, in text clearly visible and readable to the naked eye, add

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label.

Table 1

| Label 1 Assembly Number | Label 2 EVM Number |
| --- | --- |
| TIDA-050056 | TPS629210-Q1EVM |

## NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss exceeds 2%, contact the EVM coordinator for assistance.